ABSTRACT

Passwords are exposed to phishing, reuse and server breaches. FIDO2 replaces the shared secret with a key pair bound to the web site, and a hardware security key is the strongest place to hold the private key, but commercial keys are closed products. This project designed and implemented a FIDO2 authentication token on a low-cost ESP32-S3 development board. The firmware presents a USB HID interface with CTAPHID framing, implements the CTAP2 commands authenticatorGetInfo,authenticatorMakeCredential and authenticatorGetAssertion, signs with ECDSA P-256 and SHA-256 through mbedTLS, stores credentials in a dedicated flash partition and requires a button press as proof of user presence. A relying party built with FastAPI and py_webauthn issues single-use challenges and verifies every response.